-------------------------------------------------------------------------------

WITH PASSWORDS

-------------------------------------------------------------------------------

enable

config t

enable password class

no ip domain-lookup

hostname R1

line vty 0 4

password cisco

login

exit

line console 0

password cisco

login

exit

line Aux 0

password cisco

login

exit

service password-encryption

do wr

-------------------------------------------------------------------------------

NO PASSWORDS

-------------------------------------------------------------------------------

enable

terminal history size 25

config t

no ip domain-lookup

hostname R1

line vty 0 15

no password

login local

exit

line console 0

no password

login local

exit

line Aux 0

no password

login local

exit

service password-encryption

do wr