- SWITCH -

vlan 10

name Floor 1

vlan 20

name Floor 2

vlan 99

name Management

int f0/1

sw mode access

sw access vlan 10

sw nonegotiate

no shut

int g0/1

sw mode trunk

sw nonegotiate

no shut

int vlan 99

ip address 192.168.99.3 255.255.255.0

no shut

exit

ip default-gateway 192.168.1.1

hostname S1

- ROUTER -

int g0/0/0.10

description DGW VLAN 10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

int g0/0/0.20

description DGW VLAN 20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

int g0/0/0.99

description DGW for VLAN 99

encapsulation dot1Q 99

ip address 192.168.99.1 255.255.255.0

int g0/0/0

description Trunk Link to S1

no shut

exit

hostname R1