** Do Basic Config First **

exit

config t

ip domain-name cisco.com

username admin secret Cyb3rPatriot

crypto key generate rsa

1024

line vty 0 4

transport input ssh
exec-timeout 5

login local

exit

ip ssh version 2

do wr